% Exp7 Butterworth Digital (IIR) filter

clearvars;close all; clc;

% Frequency specifications:

fs = 8e3; % sampling rate 8000 Hz

fpass = 1e3;% PB edge freq 1000 Hz

fstop = 3e3;% SB edge freq 3000 Hz

% Analog to digital frequency conversion:

% normalized digital frequency in rad

omegap = 2*pi*fpass/fs;% in rad

omegas = 2*pi*fstop/fs;% in rad

% Magnitude specifications:

ap = 2;% Max Passband attenuation in dB

as = 20;% Min Stopband attenuation in dB

% Prewarping the frequencies:

OMP = 2*tan(omegap/2);% rad/sec

OMS = 2*tan(omegas/2);% rad/sec

if(OMS>OMP)

filtype = 'low';

else

filtype = 'high';

end

[N, OMC] = buttord(OMP, OMS, ap, as, 's');% Estimating the order

[b1,a1] = butter(N,1,filtype,'s');% Normalized Analog filter TF H(s)

[b,a] = butter(N,OMC,filtype,'s');% Denormalized Analog filter TF H(s)

% Applying Bilinear Transformation H(z)

[B,A]=bilinear(b,a,1);

% Plotting frequency response:

N_DFT = 2*4096;

figure(1),

% freqz(B,A);

freqz(B,A,N_DFT,fs);

title('Frequency response of Digital IIR Butterworth Filter');

grid on;

% Pole Zero Plot:

figure(2),pzmap(b1,a1);

hold on;

pzmap(b,a);

hold off;

title('Pole Zero Plot of Analog IIR Butterworth Filter');

grid on;

legend('Normalized','Denormalized');

[z,p,k] = tf2zp(B,A);

figure(3),zplane(z,p);

title('Pole Zero Plot of Digital IIR Butterworth Filter');

grid on;

% Display the parameters of the analog filter:

fprintf('Filter type: %spass filter\n',filtype);

fprintf('Order of the filter: %d\n', N);

fprintf('Cut-off frequency of the filter: %f rad/sec\n', OMC);

% Not for this AY

% Performing filtering:

% t = 2; % duration of the signal

% x = randn(1,t*fs);

% y = x;% making the copy of the signal

% % converting TF to second order sections (SOS)

% [SOS,G] = tf2sos(B,A);% G is gain

% steps = size(SOS,1);

% scale = length(G);

% for n = 1:steps

%     y = df2tr(SOS(n,1:3),SOS(n,4:6),y);

%     if(scale == 1 && n<steps)

%

%     elseif(scale == 1 && n==steps)

%         y = y*G;

%     else

%         y = y*G(n);

%     end

% end

%

% % DF-2 transposed structure:

% function y = df2tr(b,a,x)

% format long;

% len = length(x);

% s1n = 0;

% s2n = 0;

% y = zeros(size(x));

% for n=1:len

%     % compute output:

%     y(n) = (x(n)*b(1))+s1n;

%     % Update states:

%     s1n = (x(n)*b(2))-(y(n)*a(2))+s2n;

%     s2n = (x(n)*b(3))-(y(n)*a(3));

% end

% end