% Exp 8 Chebyshev - I Digital (IIR) filter

clearvars;close all; clc;

% Frequency specifications:

fs = 8e3; % sampling rate

fpass = 1e3;

fstop = 3e3;

% Analog to digital frequency conversion:

% normalized digital frequency in rad

omegap = 2*pi*fpass/fs;

omegas = 2*pi*fstop/fs;

% Magnitude specifications:

ap = 2;% Passband attenuation

as = 20;% Stopband attenuation

% Prewarping the frequencies:

OMP = 2*tan(omegap/2);

OMS = 2*tan(omegas/2);

if(OMS>OMP)

filtype = 'low';

else

filtype = 'high';

end

[N, OMC] = cheb1ord(OMP, OMS, ap, as, 's');% Estimating the order

[b1,a1] = cheby1(N,ap,1,filtype,'s');% Normalized Analog filter TF H(s)

[b,a] = cheby1(N,ap,OMC,filtype,'s');% Denormalized Analog filter TF H(s)

% Applying Bilinear Transformation H(z)

[B,A]=bilinear(b,a,1);

% Plotting frequency response:

N_DFT = 4096;

figure(1),freqz(B,A,N_DFT,fs);

title('Digital IIR Chebyshev Type-I Filter Frequency Response');

grid on;

% Pole Zero Plot:

figure(2),pzmap(b1,a1);

hold on;

pzmap(b,a);

hold off;

title('Pole Zero Plot of Analog IIR Chebyshev Type-I Filter');

grid on;

legend('Normalized','Denormalized');

[z,p,k] = tf2zp(B,A);

figure(3),zplane(z,p);

title('Pole Zero Plot of Digital IIR Chebyshev Type-I Filter');

grid on;

% Display the parameters of the analog filter:

fprintf('Filter type: %spass filter\n',filtype);

fprintf('Order of the filter: %d\n', N);

fprintf('Cut-off frequency of the filter: %f rad/sec\n', OMC);

% Performing filtering:

% t = 2; % duration of the signal

% x = randn(1,t*fs);

% y = x;% making the copy of the signal

% % converting TF to second order sections (SOS)

% [SOS,G] = tf2sos(B,A);% G is gain

% steps = size(SOS,1);

% scale = length(G);

% for n = 1:steps

%     y = df2tr(SOS(n,1:3),SOS(n,4:6),y);

%     if(scale == 1 && n<steps)

%

%     elseif(scale == 1 && n==steps)

%         y = y*G;

%     else

%         y = y*G(n);

%     end

% end

% N_DFT = length(x);

% X = fft(x,N_DFT);

% Y = fft(y,N_DFT);

% f = (0:N_DFT-1)*fs/N_DFT;

% figure(4),plot(f,20*log10(abs(X)));

% hold on;

% plot(f,20*log10(abs(Y)));hold off;

% xlim([0 fs/2]);

% grid on;

% title('Signal Spectrum');

% legend('Original','Filtered');

% % DF-2 transposed structure:

% function y = df2tr(b,a,x)

% format long;

% len = length(x);

% s1n = 0;

% s2n = 0;

% y = zeros(size(x));

% for n=1:len

%     % compute output:

%     y(n) = (x(n)*b(1))+s1n;

%     % Update states:

%     s1n = (x(n)*b(2))-(y(n)*a(2))+s2n;

%     s2n = (x(n)*b(3))-(y(n)*a(3));

% end

% end