%% FIR Type-I High Pass Filter Design using Kaiser window

% Kaiser window is a generalized window that includes -

% rectangular to Blackman window

clearvars; close all; clc;

%% Filter specifications:

fs = 16000; % Sampling frequency (Hz)

fpass = 5000; % Passband edge (Hz)

fstop = 2000; % Stopband edge (Hz)

As = 40; % Stopband attenuation (dB)

%% Input validation

if fpass <= fstop

error('For High-pass filter, fpass must be > fstop.');

end

%% Digital specifications:

wp = 2*pi*fpass/fs;

ws = 2*pi*fstop/fs;

delta_omega = wp - ws; % Transition width

%% Kaiser window parameters

if As > 50

beta = 0.1102*(As - 8.7);

elseif As >= 21

beta = 0.5842*(As - 21)^0.4 + 0.07886*(As - 21);

else

beta = 0;

end

% Estimation of filter length:

M = ceil((As - 8) / (2.285 * delta_omega));

if rem(M,2)==0, M = M + 1; end

w = kaiser(M, beta)';

fprintf('HPF Specs:\n');

fprintf('fs = %d Hz, fpass = %d Hz, fstop = %d Hz\n', fs, fpass, fstop);

fprintf('A_s = %.2f dB, Kaiser beta = %.3f, M = %d\n', As, beta, M);

%% Ideal impulse response

N = M - 1; n = 0:N; alpha = N/2;

wc = wp - delta_omega/2; % cutoff

hd = -sin(wc*(n - alpha)) ./ (pi*(n - alpha));

hd(alpha+1) = 1 - wc/pi;

b = hd .* w;

%% Frequency response:

figure(1);

freqz(b,1,4096,fs);

title('FIR High-Pass Filter (Kaiser)');

grid on;

%% Pole-zero plot:

a = [1, zeros(1,N)];% all poles are at origin

figure(2); zplane(b,a); title('Pole-Zero Plot of HPF');

grid on;

%% Test signal generation (Gaussian white noise)

rng(0);

x = randn(fs,1); x = x/max(abs(x));% Normalization

%% Pass the white noise through Designed LPF:

y = myFilter_DFI_FIR(b,x);

%% Spectrum comparison

Nfft = 8192;

f = (0:Nfft-1)*fs/Nfft;

X = fft(x,Nfft);% original signal spectrum

Y = fft(y,Nfft);% filtered signal spectrum

epsv = 1e-12;% small value to avoid NaN or Inf

magX = 20*log10(abs(X)+epsv);

magY = 20*log10(abs(Y)+epsv);

figure(3);

plot(f(1:Nfft/2), magX(1:Nfft/2),'r');

hold on;

plot(f(1:Nfft/2), magY(1:Nfft/2),'g');

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Magnitude Spectrum: Input vs Output of HPF');

legend('Input','Filtered');

grid on;

xlim([0 fs/2]);

%% Direct Form-I FIR Implementation

% For FIR LPF DF-I becomes linear convolution

% with length exactly equal to input sequence

function y = myFilter_DFI_FIR(b,x)

b = b(:); x = x(:);

M = length(b); N = length(x);

y = zeros(N,1);

for n = 1:N

acc = 0;

for k = 1:M

if (n - k + 1) > 0

acc = acc + b(k)*x(n-k+1);

end

end

y(n) = acc;

end

end